Lab 2

Date Submitted: 11/23/19

------------------------------------------------------------------------------------

Task 01:

Memory browser:

------------------------------------------------------------------------------------

Task 02:

Modified Code:

//---------------------------------------------------------------------------

// Project: Blink TM4C - CCS Lab - STARTER

//

// Author: Eric Wilbur

//

// Date: June 2014

//

//---------------------------------------------------------------------------

//------------------------------------------

// TivaWare Header Files

//------------------------------------------

#include <stdint.h>

#include <stdbool.h>

#include "inc/hw_types.h"

#include "inc/hw_memmap.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

#include <time.h>

//----------------------------------------

// Prototypes

//----------------------------------------

void hardware_init(void);

void ledToggle(void);

void delay(void);

//---------------------------------------

// Globals

//---------------------------------------

volatile int16_t i16ToggleCount = 0;

//---------------------------------------------------------------------------

// main()

//---------------------------------------------------------------------------

void main(void)

{

hardware_init();							// init hardware via Xware

while(1)									// forever loop

{

ledToggle();					// toggle LED

delay();								// create a delay of ~1/2sec

i16ToggleCount += 1;					// keep track of #toggles

}

}

//---------------------------------------------------------------------------

// hardware_init()

//

// inits GPIO pins for toggling the LED

//---------------------------------------------------------------------------

void hardware_init(void)

{

//Set CPU Clock to 40MHz. 400MHz PLL/2 = 200 DIV 5 = 40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_XTAL_16MHZ|SYSCTL_OSC_MAIN);

// ADD Tiva-C GPIO setup - enables port, sets pins 1-3 (RGB) pins for output

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

// Turn on the LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 4);

}

//---------------------------------------------------------------------------

// ledToggle()

//

// toggles LED on Tiva-C LaunchPad

//---------------------------------------------------------------------------

void ledToggle(void)

{

// LED values - 2=RED, 4=BLUE, 8=GREEN

if(GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_2))

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 0);

}

else

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 4);

}

}

//---------------------------------------------------------------------------

// delay()

//

// Creates a 500ms delay via TivaWare fxn

//---------------------------------------------------------------------------

void delay(void)

{

SysCtlDelay(13400000);		// makes delay 2x as slow!

}

------------------------------------------------------------------------------------

Lab 4

Date Submitted: 11/17/19

------------------------------------------------------------------------------------

Task 01:

Execution:

------------------------------------------------------------------------------------

Task 02:

Log:

------------------------------------------------------------------------------------

Lab 5

Date Submitted: 12/3/19

------------------------------------------------------------------------------------

Task 01:

Modified Code:

//----------------------------------------

// BIOS header files

//----------------------------------------

#include <xdc/std.h>                        //mandatory - have to include first, for BIOS types

#include <ti/sysbios/BIOS.h>                //mandatory - if you call APIs like BIOS_start()

#include <xdc/runtime/Log.h>                //needed for any Log_info() call

#include <xdc/cfg/global.h>                 //header file for statically defined objects/handles

//------------------------------------------

// TivaWare Header Files

//------------------------------------------

#include <stdint.h>

#include <stdbool.h>

#include "inc/hw_types.h"

#include "inc/hw_memmap.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

//----------------------------------------

// Prototypes

//----------------------------------------

void hardware_init(void);

void ledToggle(void);

void delay(void);

//---------------------------------------

// Globals

//---------------------------------------

volatile int16_t i16ToggleCount = 0;

//---------------------------------------------------------------------------

// main()

//---------------------------------------------------------------------------

void main(void)

{

hardware_init();                            // init hardware via Xware

BIOS_start();

}

//---------------------------------------------------------------------------

// hardware_init()

//

// inits GPIO pins for toggling the LED

//---------------------------------------------------------------------------

void hardware_init(void)

{

uint32_t ui32Period;

//Set CPU Clock to 40MHz. 400MHz PLL/2 = 200 DIV 5 = 40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5 | SYSCTL_USE_PLL | SYSCTL_XTAL_16MHZ | SYSCTL_OSC_MAIN);

// ADD Tiva-C GPIO setup - enables port, sets pins 1-3 (RGB) pins for output

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3);

// Turn on the LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3, 4);

// Timer 2 setup code

SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER2);           // enable Timer 2 periph clks

TimerConfigure(TIMER2_BASE, TIMER_CFG_PERIODIC);        // cfg Timer 2 mode - periodic

ui32Period = (SysCtlClockGet() / 2);                        // period = CPU clk div 2 (500ms)

TimerLoadSet(TIMER2_BASE, TIMER_A, ui32Period);         // set Timer 2 period

TimerIntEnable(TIMER2_BASE, TIMER_TIMA_TIMEOUT);        // enables Timer 2 to interrupt CPU

TimerEnable(TIMER2_BASE, TIMER_A);                      // enable Timer 2

}

//---------------------------------------------------------------------------

// ledToggle()

//

// toggles LED on Tiva-C LaunchPad

//---------------------------------------------------------------------------

void ledToggle(void)

{

TimerIntClear(TIMER2_BASE, TIMER_TIMA_TIMEOUT);         // must clear timer flag FROM timer

// LED values - 2=RED, 4=BLUE, 8=GREEN

if (GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_2))

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3, 0);

}

else

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 4);

}

i16ToggleCount += 1;                                    // keep track of #toggles

Log_info1("LED TOGGLED [%u] TIMES", i16ToggleCount);        // send toggle count to UIA

}

------------------------------------------------------------------------------------

Task 02:

ROV:

------------------------------------------------------------------------------------

Lab 6

Date Submitted: 12/3/19

------------------------------------------------------------------------------------

Task 01:

Modified Code:

//----------------------------------------

// BIOS header files

//----------------------------------------

#include <xdc/std.h>        //mandatory - have to include first, for BIOS types

#include <ti/sysbios/BIOS.h>    //mandatory - if you call APIs like BIOS_start()

#include <xdc/runtime/Log.h>    //needed for any Log_info() call

#include <xdc/cfg/global.h>     //header file for statically defined objects/handles

//------------------------------------------

// TivaWare Header Files

//------------------------------------------

#include <stdint.h>

#include <stdbool.h>

#include "inc/hw_types.h"

#include "inc/hw_memmap.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

//----------------------------------------

// Prototypes

//----------------------------------------

void hardware_init(void);

void ledToggle(void);

void Timer_ISR(void);

//---------------------------------------

// Globals

//---------------------------------------

volatile int16_t i16ToggleCount = 0;

//---------------------------------------------------------------------------

// main()

//---------------------------------------------------------------------------

void main(void)

{

hardware_init();    // init hardware via Xware

BIOS_start();

}

//---------------------------------------------------------------------------

// hardware_init()

//

// inits GPIO pins for toggling the LED

//---------------------------------------------------------------------------

void hardware_init(void)

{

uint32_t ui32Period;

//Set CPU Clock to 40MHz. 400MHz PLL/2 = 200 DIV 5 = 40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5 | SYSCTL_USE_PLL | SYSCTL_XTAL_16MHZ | SYSCTL_OSC_MAIN);

// ADD Tiva-C GPIO setup - enables port, sets pins 1-3 (RGB) pins for output

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3);

// Turn on the LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3, 4);

// Timer 2 setup code

SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER2);

TimerConfigure(TIMER2_BASE, TIMER_CFG_PERIODIC);

ui32Period = (SysCtlClockGet() / 2);    // period = CPU clk div 2 (500ms)

TimerLoadSet(TIMER2_BASE, TIMER_A, ui32Period);     // set Timer 2 period

TimerIntEnable(TIMER2_BASE, TIMER_TIMA_TIMEOUT);        // enables Timer 2

TimerEnable(TIMER2_BASE, TIMER_A);              // enable Timer 2

}

//---------------------------------------------------------------------------

// ledToggle()

//

// toggles LED on Tiva-C LaunchPad

//---------------------------------------------------------------------------

void ledToggle(void)

{

TimerIntClear(TIMER2_BASE, TIMER_TIMA_TIMEOUT);

// LED values - 2=RED, 4=BLUE, 8=GREEN

if (GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_2))

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3, 0);

}

else

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 4);

}

i16ToggleCount += 1;        // keep track of #toggles

Log_info1("LED TOGGLED [%u] TIMES", i16ToggleCount); // send toggle count to UIA

}

void Timer_ISR(void)

{

TimerIntClear(TIMER2_BASE, TIMER_TIMA_TIMEOUT);

Swi_post(LEDSwi);

}

------------------------------------------------------------------------------------

Task 02:

------------------------------------------------------------------------------------

Lab 7

Date Submitted: 12/3/19

------------------------------------------------------------------------------------

Task 01:

Modified Code:

//----------------------------------------

// BIOS header files

//----------------------------------------

#include <xdc/std.h>                        //mandatory - have to include first, for BIOS types

#include <ti/sysbios/BIOS.h>                //mandatory - if you call APIs like BIOS_start()

#include <xdc/runtime/Log.h>                //needed for any Log_info() call

#include <xdc/cfg/global.h>                 //header file for statically defined objects/handles

#include <xdc/runtime/Timestamp.h>          // used for Timestamp() calls

//------------------------------------------

// TivaWare Header Files

//------------------------------------------

#include <stdint.h>

#include <stdbool.h>

#include "inc/hw_types.h"

#include "inc/hw_memmap.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

//----------------------------------------

// Prototypes

//----------------------------------------

void hardware_init(void);

void ledToggle(void);

//void Timer_ISR(void);

//---------------------------------------

// Globals

//---------------------------------------

volatile int16_t i16ToggleCount = 0;

//---------------------------------------------------------------------------

// main()

//---------------------------------------------------------------------------

void main(void)

{

hardware_init();                         // init hardware via Xware

BIOS_start();

}

//---------------------------------------------------------------------------

// hardware_init()

//

// inits GPIO pins for toggling the LED

//---------------------------------------------------------------------------

void hardware_init(void)

{

//  uint32_t ui32Period;

//Set CPU Clock to 40MHz. 400MHz PLL/2 = 200 DIV 5 = 40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_XTAL_16MHZ|SYSCTL_OSC_MAIN);

// ADD Tiva-C GPIO setup - enables port, sets pins 1-3 (RGB) pins for output

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

// Turn on the LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 4);

}

//---------------------------------------------------------------------------

// ledToggle()

//

// toggles LED on Tiva-C LaunchPad

//---------------------------------------------------------------------------

void ledToggle(void)

{

static uint32_t ui32_t0, ui32_t1, ui32_t2, ui32start, ui32stop, ui32delta;  // used for Timestamp calculations

ui32_t0 = Timestamp_get32();                                    // calculate Timestamp() overhead (ui32_t2)

ui32_t1 = Timestamp_get32();

ui32_t2 = ui32_t1 - ui32_t0;

// LED values - 2=RED, 4=BLUE, 8=GREEN

if(GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_2))

{

ui32start = Timestamp_get32();                              // get starting Timer snapshot for LED benchmark

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 0);   // toggle GPIO/LED

ui32stop = Timestamp_get32();                               // get ending Timer snapshot for LED benchmark

ui32delta = ui32stop - ui32start - ui32_t2;                 // calculate LED toggle benchmark

Log_info1("LED BENCHMARK = [%u] TM4C CYCLES", ui32delta);   // send LED benchmark to Log display

}

else

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 4);

}

i16ToggleCount += 1;                                            // keep track of #toggles

Log_info1("LED TOGGLED [%u] TIMES",i16ToggleCount);             // send toggle count to UIA

}

------------------------------------------------------------------------------------

Task 02:

------------------------------------------------------------------------------------

Lab 8

Date Submitted: 12/3/19

------------------------------------------------------------------------------------

Task 01:

Modified Code:

//----------------------------------------

// BIOS header files

//----------------------------------------

#include <xdc/std.h>                        //mandatory - have to include first, for BIOS types

#include <ti/sysbios/BIOS.h>                //mandatory - if you call APIs like BIOS_start()

#include <xdc/runtime/Log.h>                //needed for any Log_info() call

#include <xdc/cfg/global.h>                 //header file for statically defined objects/handles

//------------------------------------------

// TivaWare Header Files

//------------------------------------------

#include <stdint.h>

#include <stdbool.h>

#include "inc/hw_types.h"

#include "inc/hw_memmap.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "inc/hw_ints.h"

#include "driverlib/interrupt.h"

#include "driverlib/timer.h"

//----------------------------------------

// Prototypes

//----------------------------------------

void hardware_init(void);

void ledToggle(void);

void Timer_ISR(void);

//---------------------------------------

// Globals

//---------------------------------------

volatile int16_t i16ToggleCount = 0;

//---------------------------------------------------------------------------

// main()

//---------------------------------------------------------------------------

void main(void)

{

hardware_init();                         // init hardware via Xware

BIOS_start();

}

//---------------------------------------------------------------------------

// hardware_init()

//

// inits GPIO pins for toggling the LED

//---------------------------------------------------------------------------

void hardware_init(void)

{

uint32_t ui32Period;

//Set CPU Clock to 40MHz. 400MHz PLL/2 = 200 DIV 5 = 40MHz

SysCtlClockSet(SYSCTL_SYSDIV_5|SYSCTL_USE_PLL|SYSCTL_XTAL_16MHZ|SYSCTL_OSC_MAIN);

// ADD Tiva-C GPIO setup - enables port, sets pins 1-3 (RGB) pins for output

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

// Turn on the LED

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 4);

// Timer 2 setup code

SysCtlPeripheralEnable(SYSCTL_PERIPH_TIMER2);           // enable Timer 2 periph clks

TimerConfigure(TIMER2_BASE, TIMER_CFG_PERIODIC);        // cfg Timer 2 mode - periodic

ui32Period = (SysCtlClockGet() /2);                     // period = CPU clk div 2 (500ms)

TimerLoadSet(TIMER2_BASE, TIMER_A, ui32Period);         // set Timer 2 period

TimerIntEnable(TIMER2_BASE, TIMER_TIMA_TIMEOUT);        // enables Timer 2 to interrupt CPU

TimerEnable(TIMER2_BASE, TIMER_A);                      // enable Timer 2

}

//---------------------------------------------------------------------------

// ledToggle()

//

// toggles LED on Tiva-C LaunchPad

//---------------------------------------------------------------------------

void ledToggle(void)

{

while(1)

{

Semaphore_pend(LEDSem, BIOS_WAIT_FOREVER);              // wait for Sem from ISR

// LED values - 2=RED, 4=BLUE, 8=GREEN

if(GPIOPinRead(GPIO_PORTF_BASE, GPIO_PIN_2))

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 0);

}

else

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_2, 4);

}

i16ToggleCount += 1;                                    // keep track of #toggles

Log_info1("LED TOGGLED [%u] TIMES",i16ToggleCount);     // send toggle count to UIA

}

}

//---------------------------------------------------------------------------

// Timer ISR - called by BIOS Hwi (see app.cfg)

//

// Posts Swi (or later a Semaphore) to toggle the LED

//---------------------------------------------------------------------------

void Timer_ISR(void)

{

TimerIntClear(TIMER2_BASE, TIMER_TIMA_TIMEOUT);         // must clear timer flag FROM timer

Semaphore_post(LEDSem);

}

------------------------------------------------------------------------------------

Task 02: